| Overview of the Project |  |
| --- | --- |
| - You will learn how to use a sensor input (ultrasonic ranger) to control an output (sound frequency). - You will associate various sound frequencies with the changing distance of an object relative to the Ranger. By associating certain frequencies with certain distances, you can effectively create a STEM Slide Whistle. - You will explore the relationship between distance and frequency with a linear equation. | Objectives: |
|  | - Use an input (ranger) to control an output (speaker). - Use a while loop in a program. - Define and use variables in a program. - Print variable values on the screen. - Construct a physical system (hardware) that will interact with software (your code). - Use a linear regression to determine the numeric relationship between distance (input) and sound (output). |
| Technology and Materials Needed |  |
| - 1 TI-84 CE Python Calculator - 1 TI-Innovator Hub - 1 per 3 students - 1 Grove – Ultrasonic Ranger - 1 per 3 students - Paper towel tubes - Scissors - Popsicle stick - Masking tape or hot glue - Hole Puncher - Cardstock or Manila Folder - Circle sheet (optional – check with your teacher) | TI-84 CE Python Calculator |
|  | TI-Innovator Hub |
|  | Grove - Ultrasonic Ranger Module |

Fig. 1: TI website with 10-min of Code sample programs

Learn the basics of creating, editing and running Python programs on the TI-84 Plus CE Python Graphing Calculator by following some of the 10 Minutes of Code lessons for the TI-84 Plus CE Python (link). Those lessons are a step-by step approach to introductory programming concepts. Unit 1 can especially help you become familiar with creating, editing, and running programs on the TI-84 Plus CE.

Python enables extended functions beyond the core built-in set by importing modules. For the Slide Whistle project, the module from ti.hub import * will bring in some of the additions that you will need. The import statement and functions are available on the TI Hub menus.

| Part 1: Coding the Ultrasonic Ranger |  |
| --- | --- |

Fig. 3: Selecting Program Type

Fig. 2: Starting a new Program

- Starting a new program:

Press [prgm] and then press 2:Python App to start a new Python program (Fig. 2). Since we will be using the TI Innovator Hub throughout these challenges, click on Types and select 6: Hub Project (Fig. 3).

Fig. 4: Hub Project with Modules

This will automatically import two of the

commonly used modules (Fig. 4).

- Now we need to add your ‘input’ device. Press Fns... , arrow over to Modul and select 6: ti_hub ...

Fig. 5: ti_hub module

Fig. 6: Ranger module

This will bring up all the functions available in the TI-Hub module (see Fig. 5). Select 2: Input devices ...             and 2: Ranger, which will add the Ranger module to your code.

We now need to “tell” the Hub where to find the Ranger. Press Fns... and arrow over to Modul again. On the next screen, you’ll see that the Ranger module has been added to the list of modules.

Select 8: Ranger and 1: var = ranger (“port”), as shown in Fig. 6. This will bring up a list of TI-Innovator Hub ports. Select 3: IN 1. Now it is time to define the variable we want to measure. We called it ‘dist.’ Using the [alpha] key and corresponding letters, type in your variable name.

- Use a while loop to continuously monitor the distance the Ranger reads. The ‘while not escape’ loop can be found in the ti_system module. On the TI-84 Plus CE, the [clear] key acts as an ‘escape’ key. Note that all the statements inside the ‘while not escape’ loop are indented. Use the Tools menu to add or delete indents.

- Next, we define what to measure. Select 8: Ranger and 2: var.measurement(), as shown in Fig. 6, and replace ‘var’ with the name of the variable you previously defined (in our case, ‘dist’): d=dist.measurement()

Fig. 7: Sample code for Ranger

Move to the next line. There are two ways to display the measurement on the handheld screen when running the program, print() and disp_at( ). disp_at allows the measurement, with unit, to be displayed in the line you choose. Sample code WHISTLE1 is provided in Fig. 7.

print() can be found by pressing Fns... and  I/O, while disp_at( ) is located under the modules, 4: ti_system module, 7: disp_at ().

Fig. 8: Ranger connected to Hub in IN1

Plug the Ranger into “IN 1” on your TI-Innovator Hub and connect the Hub to your TI-84 Plus CE Python. Run the program by pressing TRACE Run.

- Move the Ranger around you to see how the distance changes. The Ranger’s range is between 2 to 350 cm (0.02 m to 3.5 m) and has a 1 cm resolution (sensor’s ability to detect a change).

- Can you determine the default unit of measure on the calculator?

| Part 2: Coding the Sound |  |  |
| --- | --- | --- |
| Fig. 9: ti_hub module Fig. 10: ti_hub import * - Starting a New Program As we did in part 1, press [prgm] and then press 2 :Python App to start a new Python program (Fig. 2). Press Fns... and arrow over to Modul again. Select 6: ti_hub ... , arrow over to ‘Advanced’ and press 1:from ti_hub import * (Fig.. 10). Fig. 11: Selecting variable ‘Type’ Fig. 12: Select ‘input’ request - In this step, we will define two variables, ‘freq’ and ‘time.’ - Press Fns... and arrow over to Type and select 2:float(). Press Fns... again and arrow over to I/O and select 2:input(). - Using the [alpha] key, type in the ‘request’ for frequency and add the variable name ‘freq’ and an = sign in front of the request. freq=float(input("What frequency?")) - Using the [alpha] key, type in the ‘request’ for the duration and add the variable name ‘time’ and an = sign in front of the request. time=float(input("For how long?" )) Fig. 13: Selecting ‘sound’ module Fig. 14: Type of sound output We now need to insert a sound command. Press Fns... and arrow over to Modul again. Select 6: ti_hub ... , 1: Hub Built-in devices, and 3: Sound. This will add the sound module to your code. - To add the type of sound output, press Fns... and arrow over to Modul again. The ‘Sound’ module is now available. Select 8: Sound ... and 1: tone (freq, time) as in Fig. 14. Fig. 15: Frequency and Duration of sound output - Run your program. When prompted, enter one of the sample frequencies from the “Piano Key Reference Sheet.” For time, enter a value in seconds (ex. 3 would equal 3 second duration that the frequency will play). See Fg. 15 for an example. |  |  |
| Part 3: Building the Slide Whistle |  |  |

- Trace the slot to be cut on the paper towel tube with the popsicle stick.

- When creating the slot for the slide, use a hole puncher if available. This way there is a “pilot hole” for the scissors to easily cut the length of the tube.

The slot should look similar to the image shown to the right.

- Next, use one end of the tube to trace a circle using cardstock or oak tag paper.

Your teacher may have opted to print out the circle sheet which prints out 20 circles at once. They are ~ 1.5” in diameter, which is the diameter of a typical paper towel tube. Use one of those and make sure it fits inside your paper towel role.

- Now tape the circle onto the popsicle stick. This will be the “slide” part of your slide whistle.

- Insert the slide into the tube. The circle should fit inside the tube with the sides just touching. This will be used to change the distance that the Ranger reports in your program.

- Tape the Ranger onto the end of the tube.

- Make sure the Ranger is plugged into IN 1 on the TI-Innovator Hub.

Part 4: Calibrate the Slide Whistle

| Distance (m) | Frequency (Hz) |
| --- | --- |
| .22 | 263 |
| .013 | 932 |

Fig. 17: Data Table

Fig. 16: First value reading – Lowest tone/frequency

Move the slide as far away from the Ranger as possible. This will be the first value reading – it will correspond to the lowest tone/frequency.

- Now run your Ranger program. Pick a consistent distance value from the list that appears on the screen.

Fig. 18: Second value reading – Highest tone/frequency

Enter that value in the first cell of the table in Fig. 17. Add a low frequency in the cell next to it.

- To get the highest tone/frequency, move the slide as close to the Ranger as possible.

- Now run your Ranger program again. Pick a consistent distance value from the list that appears on the screen.

- Enter that value in the second cell of the table in Fig. 17. Add a high frequency to the cell next to it.

Fig. 19: Distance and Frequency values in Calculator Lists

- Exit your Ranger program and press the s t a t  key. Select 1:Edit and enter your values from the table above in d   and e  .

Fig. 20: Select lists and location for LinReg

- Exit out of the lists and press the s t a t key again. Select CALC and    4: LinReg (ax+b). On the next screen, be sure to select the lists where you stored the data.

- You can store the regression equation in Y1 by pressing the v key, selecting Y-VARS, 1: Function and 1: Y1 (see Fig. 20).

- You might want to write down your values for a and b here:

a = _________        b = _________

Now it is time to get back to our python code. Our final program combines steps from the first two. One way to do that is to go back to the first program we had called WHISTLE1 and replicate it with a new name.

- Go to File Manager, select your program, press Manage and 1: Replicate Program .... Assign a new name. We called ours WHISTLE3.

- Following the same way we did it in part 2, we now need to insert a sound command. Press Fns... and arrow over to Modul again. Select 6: ti_hub ... , 1: Hub Built-in devices, and 3: Sound. This will add the sound module to your code.

Fig. 21: Sample program WHISTLE3

OPTION 1:

- Now we add our ‘new’ definition of frequency by adding a line “freq=m*d+b” which means the frequency of the sound will be equal to the slope times the distance (x-value on the regression) + the y-intercept.

- To add the type of sound output, press Fns... and arrow over to Modul again. The ‘Sound’ module is now available. Select 8: Sound ... and 1: tone (freq, time) as in Fig. 14.

  - Before running the program, you need to replace the variables ‘m’ and ‘b’ with the numeric values you recorded in # 10 above.

- The line “sound.tone(freq,.1)” will play that value for .1 seconds at a time. As the distance of your slide changes, so will the frequency! (See sample code in Fig. 21)

Fig. 22: Sample code for Option 2

OPTION 2:

- With some minor modification to the code above, we can actually recall the regression equation directly and then use it in the ‘definition’ of our frequency.

- With the curser just below the line from ti_system import *, press Fns... and arrow over to Modul . Select 4: ti_system ... and then 4:var=recall_RegEq().

- You can keep the line freq = m*d+b in the code, just ‘comment’ it out (see Fig. 22).

- Sample code WHISTLJH.8xv is shown in Fig. 22.

- Now run the program and make some music using Python code and math!