| Function | Example | Behavior |
| --- | --- | --- |
| from module_name import * | from ti_hub import * | Imports all the functions in the ti_hub module for use in the program. The ti_hub module includes all the necessary additions needed for the Path to STEM Projects. |
| var=analog_in(“port”) | potentiometer=analog_in("BB 5") | Controls an input device through port BB5 on the breadboard. In the first example, it associates port BB5 with the name, “potentiometer” (a potentiometer is attached to the breadboard). To find this in the menu, go to [menu]/8. TI Hub/3. Add Input Device/8. Analog In |
| for index in range(stop value): block | for n in range(10): | Repeats the statements in the block 10 times, If n is less than the stop value, 10, the loop continues to repeat. The block starts with a colon and includes the indented lines that follow. |
| var.range(min,max) | electrode.range(0,100) | The range option allows a user to set a range of numeric values for a sensor or device between a user defined minimum value and a maximum value. Usually this is used to scale the readings of a sensor. To access it, type a decimal after a variable and range will be an option in a popup menu. |
| text_at(vertical position,”text”,”horizontal position”) | text_at(6,”RED”,”left”) | text_at is similar to “print” but enables a user to specify the row that the text should display on the calculator screen (1 through 8), and the horizontal position of the message (right, center, left). |

See TI-Innovator Hub Technology eGuide for more background on Hub-specific commands – Link

See TI-Nspire CXII Python Programming eGuide for more background on Python commands - Link